| Laboratory 2: Integration of Sensors with ESP32 | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives:

- Learn how to wire the DHT11 and MQ-2 sensors to the ESP32 board.

- Understand the basic setup and configuration using ESPHome.

- Test and view sensor data through Home Assistant.

Experiment 1: Integration of Sensors with ESP32

Equipment:

Quantity

ESP32	1

Fan Motor	1

DHT11 Sensor	1

- Wiring the Sensors to the ESP Board

- DHT11 (Temperature and Humidity Sensor)

• VCC → 3.3V

• GND → GND

• DATA → GPIO22

- Fan Motor

• VCC → 3.3V

• GND → GND

• DATA → GPIO25

- Open ESPHome and Select the Device

  - Go to the ESPHome Dashboard

  - Select your device (e.g., sensor)

  - Click EDIT to modify the YAML configuration

- Add Code for the Sensors in the YAML File

Add the following to your configuration file:

  - DHT11 (Temperature and Humidity Sensor)

sensor:

- platform: dht

pin: 22

model: DHT11

temperature:

name: "Temperature"

id: temperature

unit_of_measurement: "°C"

humidity:

name: "Humidity"

id: humidity

unit_of_measurement: "%"

update_interval: 10s

  - Fan Motor

output:

- platform: ledc

pin: 25

id: fan_pwm

frequency: 25000 Hz

inverted: true

fan:

- platform: speed

name: "DC Fan"

output: fan_pwm

speed_count: 100

restore_mode: ALWAYS_ON

- Save and Install the Firmware

- Click SAVE

- Click INSTALL

- Choose Wirelessly or USB depending on your setup

- Wait for the upload to complete and let the device reboot

- View Sensor Data in Home Assistant

- Go to your Home Assistant Overview Dashboard

- You should now see sensor readings such as:

- Temperature / Humidity from DHT11

- Gas Detection Level from MQ-2

Take 1: Design and Build an Automated Fan Control System Using Temperature and Humidity Sensors.

Requirements:

1. Use an ESP32/ESP8266 board and a DHT11 sensor.

2. Connect a DC fan controlled via PWM (LEDC) to allow variable fan speed.

3. Read temperature and humidity every 10 seconds.

4. Combine the sensor reading and fan control code into a single ESPHome YAML file.

5. mplement automation so the fan operates according to the following conditions:

• Temperature ≥ 30°C → Fan runs at 100% (full speed)

• Temperature < 27°C → Fan turns OFF

6. Display temperature and humidity values in Home Assistant.

Home Assistant Dashboard:

Code:

----- Have a good day -----

Answer:

sensor:

- platform: dht

pin: 22

model: DHT11

temperature:

name: "Room Temperature"

id: temperature

unit_of_measurement: "°C"

humidity:

name: "Room Humidity"

id: humidity

unit_of_measurement: "%"

update_interval: 10s

output:

- platform: ledc

pin: 25

id: fan_pwm

frequency: 25000 Hz

inverted: true

fan:

- platform: speed

name: "DC Fan"

id: dc_fan

output: fan_pwm

speed_count: 100

restore_mode: ALWAYS_OFF

interval:

- interval: 10s

then:

- lambda: |-

if (id(temperature).has_state()) {

float t = id(temperature).state;

if (t >= 30.0) {

id(dc_fan).turn_on();        // เปิดพัดลม

id(dc_fan).speed = 1.0;      // ตั้งความเร็ว 100%

} else if (t < 27.0) {

id(dc_fan).turn_off();       // ปิดพัดลม

}

}