| Laboratory 3: Integration of Sensors with ESP32 | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives

- Connect multiple sensors (LDR, LED, PIR) to the ESP32 board.

- Write configuration in ESPHome to read sensor data and control the LED.

- Display sensor data on Home Assistant.

Experiment 1: Integration of Sensors with ESP32

Equipment:

|  | Quantity |
| --- | --- |
| ESP32 | 1 |
| MQ-2 | 1 |
| Active Buzzer | 1 |

- Wiring the Sensors to the ESP Board

- MQ-2

  - VCC → 5V

  - GND → GND

  - A0 → GPIO34 (ADC input)

  - D0 → GPIO17 (digital input)

- Active Buzzer

  - VCC → VCC

  - AOUT → GPIO21

  - GND → GND

2. Open ESPHome and Select the Device

  - Go to the ESPHome Dashboard

  - Select your device (e.g., sensor)

  - Click EDIT to modify the YAML configuration

3. Add Code for the Sensors in the YAML File

Add the following to your configuration file:

- MQ-2

sensor:

- platform: adc

pin: GPIO34

name: "MQ-2 Gas Sensor Analog"

id: mq2_sensor

update_interval: 2s

attenuation: 12db

filters:

- multiply: 3.3

binary_sensor:

- platform: gpio

pin: GPIO17

name: "MQ-2 Gas Sensor Digital"

id: mq2_digital

# Buzzer Module Active

output:

- platform: gpio

pin: GPIO21

id: buzzer_output

inverted: true

switch:

- platform: output

name: "Buzzer"

output: buzzer_output

4. Save and Install the Firmware

  - Click SAVE

  - Click INSTALL

  - Choose Wirelessly or USB depending on your setup

  - Wait for the upload to complete and let the device reboot

5. View Sensor Data in Home Assistant

  - Go to your Home Assistant Overview Dashboard

  - You should now see sensor readings such as:

• Gas Detection Level from MQ-2

•	Buzzer Module Active

Take 1: Design and build a gas alarm system using MQ-2 sensors and buzzers.

Requirements:

1. Use an ESP32/ESP8266 board and an MQ-2 Gas Sensor.

2. Connect an Active Buzzer to indicate gas levels.

3. Read MQ-2 Analog values every 2 seconds.

4. Combine the sensor reading and buzzer control code into a single ESPHome YAML file.

5. Implement automation so the buzzer operates according to the following conditions:

• MQ-2 Analog ≥ 1.5V → Buzzer turns ON (alert)

• MQ-2 Analog < 1.5V → Buzzer turns OFF

6. Display MQ-2 Analog values (and optionally Digital output) in Home Assistant.

Continued on the next page.

CODE:

----- Have a good day -----

ANSWER:

# ---------- MQ-2 Analog ----------

sensor:

- platform: adc

pin: GPIO34

name: "MQ-2 Gas Sensor Analog"   # ชื่อเต็ม

id: mq2_sensor

update_interval: 2s

attenuation: 12db

filters:

- multiply: 3.3   # แปลงค่าเป็นโวลต์

# ---------- Active Buzzer ----------

output:

- platform: gpio

pin: GPIO21

id: buzzer_output

inverted: true

switch:

- platform: output

name: "Active Buzzer"

output: buzzer_output

# ---------- Automation ----------

interval:

- interval: 1s

then:

- lambda: |-

const float threshold = 2;  // กำหนดค่ามาตรฐาน

if (id(mq2_sensor).state > threshold) {

id(buzzer_output).turn_on();   // เกินค่า → ดัง

} else {

id(buzzer_output).turn_off();  // ไม่เกิน → เงียบ

}