| Laboratory 4: Integration of Sensors with ESP32 | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives

- Connect a Servo Motor and Buzzer to the ESP32 board.

- Configure ESPHome to control these components.

- Monitor and control the devices through Home Assistant.

Experiment 1: Integration of Sensors with ESP32

Equipment:

|  | Quantity |
| --- | --- |
| ESP32 | 1 |
| Motion Sensor Detector Module HC-SR501 | 1 |
| LDR Photosensitive Sensor Module Light Sensor | 1 |

- Wiring the Sensors to the ESP Board

- Motion Sensor Detector Module HC-SR501

  - VCC → 3V

  - GND → GND

  - OUT → GPIO14

# LDR Photosensitive Sensor Module Light Sensor

  - VCC → 3V

  - GND → GND

  - OUT → GPIO14

2. Open ESPHome and Select the Device

- Go to the ESPHome Dashboard

- Select your device (e.g., sensor)

- Click EDIT to modify the YAML configuration

3. Add Code for the Sensors in the YAML File

Add the following to your configuration file:

- Motion Sensor Detector Module HC-SR501

# PIR Motion Sensor

binary_sensor:

- platform: gpio

pin:

number: GPIO14

mode: INPUT

name: "Living Room Motion"

device_class: motion

# LDR Photosensitive Sensor Module Light Sensor

# ----------------------------

# Analog LDR (AO)

# ----------------------------

sensor:

- platform: adc

pin: GPIO35

name: "Living Room Light (Analog)"

update_interval: 10s

attenuation: 11db

filters:

- multiply: 3.3

# ----------------------------

# Digital LDR (DO)

# ----------------------------

binary_sensor:

- platform: gpio

pin: GPIO26

name: "Bright Light Detected (Digital)"

device_class: light

filters:

- delayed_on: 100ms

- delayed_off: 100ms

4. Save and Install the Firmware

- Click SAVE

- Click INSTALL

- Choose Wirelessly or USB depending on your setup

- Wait for the upload to complete and let the device reboot

5. View Sensor Data in Home Assistant

- Go to your Home Assistant Overview Dashboard

- You should now see sensor readings such as:

- Servo Motor (PWM)

- Buzzer

Take 1: Design an LED control system using a light sensor (LDR) to automatically turn ON the LED when it is dark and OFF when it is bright.

Requirements:

1. Use an ESP32 board and an LDR sensor (Analog + Digital).

2. Connect an LED to indicate light/dark condition.

3. Read LDR Analog values every 1 second.

4. Combine the Analog/Digital sensor reading and LED control code into a single ESPHome YAML file.

5. Implement automation so the LED operates according to the following conditions:

• LDR Analog < 2.0V → LED turns ON (dark)

• LDR Analog ≥ 2.0V → LED turns OFF (bright)

Continued on the next page.

CODE:

----- Have a good day -----

ANSWER:

# LED Output

output:

- platform: gpio

pin: GPIO13

id: led_output

# Analog LDR

sensor:

- platform: adc

pin: GPIO34

name: "LDR Analog"

id: ldr_analog

update_interval: 1s

attenuation: 11db

filters:

- multiply: 3.3  # ปรับค่าเป็นโวลต์ถ้าต้องการ

# Digital LDR

binary_sensor:

- platform: gpio

pin:

number: GPIO26

mode: INPUT

inverted: false

name: "LDR Digital"

id: ldr_digital

device_class: light

# Automation: LED control based on LDR analog

interval:

- interval: 1s

then:

- if:

condition:

sensor.in_range:

id: ldr_analog

above: 2.0   # ปรับตามสภาพแสง

then:

- output.turn_off: led_output

else:

- output.turn_on: led_output