| Laboratory 5: Creating a Smart Home Dashboard in Home Assistant (Lovelace UI) | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives

- Learn how to design a Lovelace Dashboard to visualize real-time sensor data.

- Understand how to use various Lovelace cards such as Entities, Graph, Gauge, and Button.

- Practice integrating data from sensors and actuators to build an interactive Smart Home UI.

## Equipment and Software

| Item | Description |
| --- | --- |
| 💻 Computer | Running Docker |
| 🏠 Home Assistant (Container) | Core system for Smart Home dashboard |
| 📡 ESP32 / ESP8266 | Microcontroller board for sensor data |
| 🌡️ DHT11 / DHT22 | Temperature and humidity sensor |
| 💨 MQ-2 Gas Sensor | Detects smoke and gas |
| 💡 LDR Sensor | Measures light intensity |
| 🚶 PIR Motion Sensor (HC-SR501) | Detects motion |
| 🔔 Buzzer / 💨 Fan / 💡 LED | Actuators controlled via GPIO or relay |

### Enable YAML Mode for Lovelace Dashboard

### Edit the file configuration.yaml in your /config/ directory and add:

Yaml :

lovelace:

mode: yaml

resources:

- url: /hacsfiles/lovelace-card-mod/card-mod.js

type: module

dashboards:

smart-home-dashboard:

mode: yaml

title: "Smart Home Dashboard"

filename: lovelace-dashboard.yaml

icon: mdi:home-assistant

💡 Note:

- Restart Home Assistant after editing.

- The dashboard file lovelace-dashboard.yaml will be created and stored in /config/.

- The dashboard name must contain a hyphen (-) instead of an underscore (_).

Example: ✅ smart-home-dashboard

❌ smart_home_dashboard

- After saving, go to

Settings → Developer Tools → YAML → Check Configuration and then click Restart if no errors appear.

- Create the Dashboard File

- Create a new file named lovelace-dashboard.yaml in the /config/ directory.

- Paste the code below:

Yaml:

title: "🏠 Smart Home Dashboard"

views:

- title: "Living Room"

icon: mdi:sofa

cards:

# --- Sensor Data ---

- type: entities

title: "📋 Sensor Data"

entities:

- entity: sensor.living_room_temperature

name: "🌡️ Temperature"

- entity: sensor.living_room_humidity

name: "💧 Humidity"

- entity: sensor.mq_2_gas_level

name: "💨 Gas Level"

- entity: sensor.ldr_light_level

name: "💡 Light Level"

- entity: binary_sensor.living_room_motion

name: "🚶 Motion Detected"

- entity: binary_sensor.gas_alarm_active

name:"⚠️ Gas Alarm"

# --- Historical Graphs ---

- type: history-graph

title: "📈 Environment Trends"

hours_to_show: 12

refresh_interval: 30

entities:

- entity: sensor.living_room_temperature

- entity: sensor.living_room_humidity

- entity: sensor.mq_2_gas_level

# --- Temperature Gauge ---

- type: gauge

entity: sensor.living_room_temperature

name: "🌡️ Temperature °C"

min: 0

max: 50

severity:

green: 0

yellow: 30

red: 40

# --- Light Level Gauge ---

- type: gauge

entity: sensor.ldr_light_level

name: "💡 Light Level (V)"

min: 0

max: 3.3

severity:

red: 0.5

yellow: 1.0

green: 2.0

# --- Device Controls ---

- type: grid

title: "🔧 Device Controls"

columns: 3

cards:

- type: button

entity: fan.living_room_fan

name: "Fan"

icon: mdi:fan

tap_action: { action: toggle }

- type: button

entity: switch.active_buzzer

name: "Buzzer"

icon: mdi:volume-high

tap_action: { action: toggle }

- type: button

entity: switch.indicator_led

name: "LED"

icon: mdi:led-on

tap_action: { action: toggle }

# --- Map (for Lab 9) ---

- type: map

title: "📍 Sensor Location"

default_zoom: 15

entities:

- entity: sensor.living_room_temperature

name: "Living Room Sensor"

- Add Automations for Device Control

- Edit /config/automations.yaml and add:

Yaml:

# Turn on the fan when temperature exceeds 30°C

- alias: "Auto Fan Control"

trigger:

- platform: numeric_state

entity_id: sensor.living_room_temperature

above: 30

action:

- service: fan.turn_on

entity_id: fan.living_room_fan

# Turn on buzzer and LED when gas is detected

- alias: "Gas Warning Alert"

trigger:

- platform: state

entity_id: binary_sensor.gas_alarm_active

to: "on"

action:

- service: switch.turn_on

entity_id: switch.active_buzzer

- service: switch.turn_on

entity_id: switch.indicator_led

- service: persistent_notification.create

data:

title: "Gas Alert 🚨"

message: "⚠️ Gas or smoke detected in the room!"

# Automatically turn on the light when motion detected in darkness

- alias: "Auto Light When Motion"

trigger:

- platform: state

entity_id: binary_sensor.living_room_motion

to: "on"

condition:

- condition: numeric_state

entity_id: sensor.ldr_light_level

below: 0.7

action:

- service: light.turn_on

entity_id: light.room_light

- View the Dashboard

Open:

👉 http://localhost:8123/lovelace/smart-home-dashboard

You should see a clean, interactive dashboard containing:

• Real-time sensor readings

• Graphs and gauges

• Control buttons for fan, buzzer, and LED

• A map card (for use in Lab 9)

Take 1: Organize and Add a “Bedroom” View (Bonus Task)

Requirements

1. Add a new view named Bedroom to your existing YAML dashboard.

The view must include:

• One Entities Card showing at least: sensor.bedroom_temperature, sensor.bedroom_humidity, and binary_sensor.bedroom_motion

• At least one Gauge Card (for example, temperature)

• At least one Grid Card with two control buttons (e.g., Light and Buzzer)

2. Organize your dashboard layout for clarity and readability:

• Group cards logically into Sensor Data, Graph/Visualization, and Device Controls sections.

• Use clear titles, icons, and consistent styling for each view (Living Room & Bedroom).

3. (Optional — Bonus) Use card-mod styling or other Lovelace resources to improve spacing or hide unwanted headers.

• Make sure the resource is declared properly under lovelace: resources: in your YAML configuration.

• Restart Home Assistant after updating the configuration.

4. Submission Requirements:

• Paste your updated lovelace-dashboard.yaml containing both Living Room and Bedroom views into a Notepad file.

• Capture and submit screenshots of the Bedroom dashboard showing your entities and control buttons working.

Home Assistant Dashboard:

Continued on the next page.

Code:

----- Have a good day -----

Answer:

- title: "Bedroom"

icon: mdi:bed

cards:

- type: entities

title: "📋 Bedroom Sensor Data"

entities:

- entity: sensor.bedroom_temperature

name: "🌡 Temperature"

- entity: sensor.bedroom_humidity

name: "💧 Humidity"

- entity: binary_sensor.bedroom_motion

name: "🚶 Motion"

- type: gauge

entity: sensor.bedroom_temperature

name: "🌡 Temp °C"

min: 0

max: 50

severity:

green: 0

yellow: 30

red: 40

- type: grid

title: "🔧 Bedroom Controls"

columns: 2

cards:

- type: button

entity: light.bedroom_light

name: "Light"

icon: mdi:lightbulb

tap_action:

action: toggle

- type: button

entity: switch.bedroom_buzzer

name: "Buzzer"

icon: mdi:volume-high

tap_action:

action: toggle