| LAB 6: Smart Notification System using Telegram and Home Assistant | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives

- To configure Telegram Bot with Home Assistant for real-time alert notifications.

- To create automations that trigger alerts when sensors detect abnormal conditions.

- To demonstrate a simple IoT smart home notification system.

## Equipment and Software

| Item | Description |
| --- | --- |
| 💻 Home Assistant (Docker) | Main automation server |
| 🧠 ESPHome + ESP32 | Microcontroller with sensors |
| 🌡️ DHT11 | Temperature and Humidity sensor |
| 💨 MQ-2 | Gas and smoke sensor |
| 🚶 HC-SR501 | PIR motion sensor |
| 🔔 Buzzer / LED | Alert indicators |
| 🤖 Telegram App | Notification platform |

Step 1 – Download and Install Telegram

Before creating your Telegram Bot, make sure the Telegram application is installed on your device.

- For Mobile (Recommended)

1. Visit https://www.telegram.org

2. Choose Android or iPhone / iPad.

3. Tap Download Telegram to open the Play Store or App Store.

4. Install the app and open it after installation.

5. Sign in using your phone number and verification code.

- For Desktop (Windows / macOS / Linux)

1. Go to https://www.telegram.org or the main site’s Get Telegram for PC / Mac section.

2. Select your operating system (Windows, macOS, or Linux).

3. Download the installer file and follow the on-screen instructions.

4. After installation, log in by scanning the QR code with your mobile Telegram app or entering your phone number.

💡Tip

• Using the same Telegram account on both mobile and desktop lets you receive Home Assistant alerts anywhere.

• Once Telegram is installed and logged in, you can proceed to Step 1 – Create a Telegram Bot from the Lab 6 document.

Step 2 – Create a Telegram Bot

- Open Telegram → search for @BotFather.

- 👉 Copy the API Token — it will be used in Home Assistant.

Step 3 – Start the Bot Conversation

⚠️ This step is required — otherwise Telegram will block all messages sent from Home Assistant.

- Open the link of your bot, e.g.

https://t.me/SmartHomeBubuNotifierBot

- Click Start

- Once it replies “You started this bot”, it’s ready for use ✅

Step 4 – Get Your Chat ID

- In Telegram, search for @userinfobot.

- Click Start, and you will receive a message like:

➤  This number is your Chat ID.

Step 5 – Configure Telegram in Home Assistant

- Edit your configuration.yaml file and add the following block at the end:

Yaml:

# ==============================

# 🔔 Telegram Notification Setup

# ==============================

telegram_bot:

- platform: polling

api_key: "xxxxxxxxxxxxxxxxxxxxxxxxxxxx "   # ← Replace with your own Token

allowed_chat_ids:

- 987654321   # ← Replace with your own Chat ID

notify:

- name: telegram

platform: telegram

chat_id: 987654321

💡 Tip: For better security, store the token and chat ID in secrets.yaml like:

Step 6 – Check Configuration and Restart

- Go to Developer Tools → YAML → Check Configuration

- If no errors appear → Go to Settings → System → Restart

Step 7 – Test Telegram Notification

- Go to Developer Tools → Actions

- Paste this YAML snippet:

Yaml:

action: notify.telegram

target: {}

data:

message: "✅ Test message from Home Assistant!"

- Click Perform action

📱 If you see a message in Telegram saying

“✅ Test message from Home Assistant!”

then your integration works perfectly 🎉

Step 8 – Create Automation Rules

- Open the file /config/automations.yaml and add these automations:

Yaml:

# =========================================

# 🚨 Automation 1: Gas Detection Alert

# =========================================

- alias: "Alert: Gas Detected"

trigger:

- platform: state

entity_id: binary_sensor.gas_alarm_active

to: "on"

action:

- service: notify.telegram

data:

message: "⚠️ Gas detected! Please check immediately!"

- service: switch.turn_on

entity_id: switch.active_buzzer

- delay: "00:00:10"

- service: switch.turn_off

entity_id: switch.active_buzzer

# =========================================

# 🚶 Automation 2: Motion Detection Alert

# =========================================

- alias: "Alert: Motion Detected"

trigger:

- platform: state

entity_id: binary_sensor.living_room_motion

to: "on"

action:

- service: notify.telegram

data:

message: "🚶 Motion detected in the living room!"

# =========================================

# 🌡️ Automation 3: High Temperature Alert

# =========================================

- alias: "Alert: High Temperature"

trigger:

- platform: numeric_state

entity_id: sensor.living_room_temperature

above: 32

action:

- service: notify.telegram

data:

message: >

🔥 High temperature alert!

Current temperature: {{ states('sensor.living_room_temperature') }} °C

- After saving the file, reload automations:

Yaml:

action: automation.reload

target: {}

data: {}

Take 1: Combined Smart Alert (Bonus)

Requirements

- Create one automation that sends a detailed summary report to Telegram.

whenever any of the following occurs:

• Gas alarm ON

• Motion detected

• Temperature > 32 °C

- The message must include:

• Current values of all sensors

• Timestamp ({{ now().strftime('%H:%M:%S') }})

• Room location

- Use multiline message format (>)

- Test the automation by simulating one condition

Continued on the next page.

Code:

----- Have a good day -----

Answer:

- alias: "Smart Combined Alert"

trigger:

- platform: state

entity_id: binary_sensor.gas_alarm_active

to: "on"

- platform: state

entity_id: binary_sensor.living_room_motion

to: "on"

- platform: numeric_state

entity_id: sensor.living_room_temperature

above: 32

action:

- service: notify.telegram

data:

message: >

🚨 *Smart Home Alert*

🕒 Time: {{ now().strftime('%H:%M:%S') }}

🌡 Temperature: {{ states('sensor.living_room_temperature') }} °C

💨 Gas Level: {{ states('sensor.mq_2_gas_level') }}

🚶 Motion: {{ states('binary_sensor.living_room_motion') }}

📍 Location: Living Room

✅ Please check your environment immediately.