| Laboratory 7: Location-Based Smart Notification using Telegram + Home Assistant | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives

## Enable Home Assistant to send Telegram alerts with embedded Google Maps location links.

## Practice using templated automation messages in Home Assistant.

## Allow users to know where an event occurred - e.g., Kitchen, Living Room, Bedroom.

## Demonstrate a location-aware Smart Home alert system.

## Equipment and Software

| Item | Description |
| --- | --- |
| 💻 Home Assistant (Docker) | Main automation and control system |
| ⚙️ ESP32 + ESPHome | Microcontroller connected to sensors |
| 🌡️ DHT11 | Temperature and humidity sensor |
| 💨 MQ-2 | Gas/smoke detection sensor |
| 🚶 HC-SR501 | Motion detection sensor |
| 🔔 Active Buzzer / LED | Alert device |
| 🤖 Telegram App | Notification receiver |
| 🗺️ Google Maps | Used to view sensor locations |

System Concept

When an event occurs (e.g., gas leak, motion, or high temperature), Home Assistant sends a Telegram message with a Google Maps link showing the sensor’s location.

Example Telegram message:

⚠️ Gas leak detected in the kitchen.

📍 See location on map.

### 1. Define Sensor Coordinates

### Assign fixed GPS coordinates for each sensor location:

| Location | Latitude | Longitude |
| --- | --- | --- |
| Living Room | 13.9123 | 100.5211 |
| Kitchen | 13.9125 | 100.5213 |
| Bedroom | 13.9126 | 100.5215 |

💡 Tip: To find coordinates, open Google Maps, right-click on a location → “What’s here?” → copy latitude and longitude.

2. Add Automation Rules in Home Assistant

- Edit /config/automations.yaml and add the following:

Yaml:

# =========================================

# 🚨 Gas Detection with Location

# =========================================

- alias: "Alert: Gas Detected with Location"

trigger:

- platform: state

entity_id: binary_sensor.gas_alarm

to: "on"

action:

- service: notify.telegram

data:

message: >

⚠️ Gas detected in Kitchen!

📍 [View on Map](https://maps.google.com/?q=13.9125,100.5213)

- service: switch.turn_on

entity_id: switch.active_buzzer

- delay: "00:00:10"

- service: switch.turn_off

entity_id: switch.active_buzzer

# =========================================

# 🚶 Motion Detection with Location

# =========================================

- alias: "Alert: Motion Detected with Location"

trigger:

- platform: state

entity_id: binary_sensor.living_room_motion

to: "on"

action:

- service: notify.telegram

data:

message: >

🚶 Motion detected in Living Room!

📍 [View on Map](https://maps.google.com/?q=13.9123,100.5211)

# =========================================

# 🌡️ High Temperature with Location

# =========================================

- alias: "Alert: High Temperature with Location"

trigger:

- platform: numeric_state

entity_id: sensor.living_room_temperature

above: 32

action:

- service: notify.telegram

data:

message: >

🔥 High temperature detected!

Current: {{ states('sensor.living_room_temperature') }} °C

📍 [View on Map](https://maps.google.com/?q=13.9126,100.5215)

3. Validate and Reload Automations

- Go to Developer Tools → YAML → Check Configuration to ensure no syntax errors.

- Then go to Developer Tools → Actions and execute:

Yaml:

action: automation.reload

target: {}

data: {}

- Once reloaded, your new automations are active ✅

Take 1: Design and Implement a Smart Unified Notification System for Multiple Events

Objective:

To design and implement a location-aware smart notification system in Home Assistant that can detect multiple simultaneous events (Gas Leak, Motion, and High Temperature) and send a single summarized alert to Telegram — instead of multiple separate messages.

Requirements:

- Hardware and Platform

• Use an ESP32 or ESP8266 board integrated with ESPHome.

• Use the following sensors:

• MQ-2 gas sensor → detects gas or smoke.

• HC-SR501 motion sensor → detects movement.

• DHT11 sensor → measures temperature and humidity.

• All sensors must be connected and registered in Home Assistant.

- Automation and Notification

• Create a single automation in Home Assistant that combines all three triggers:

• Gas sensor (binary_sensor.gas_alarm)

• Motion sensor (binary_sensor.living_room_motion)

• Temperature sensor (sensor.living_room_temperature)

• The automation should:

• Wait for 5 seconds to ensure all events occurring close in time are grouped.

• Collect current values of all sensors.

• Send one unified message to Telegram that summarizes all detected events.

• Include the current time and Google Maps location link of the affected area.

Code:

----- Have a good day -----

Answer:

- alias: "Smart Unified Alert"

description: "Combine gas, motion, and temperature events into one Telegram message"

mode: single

trigger:

- platform: state

entity_id:

- binary_sensor.gas_alarm

- binary_sensor.living_room_motion

- platform: numeric_state

entity_id: sensor.living_room_temperature

above: 32

action:

- delay: "00:00:05"

- service: notify.telegram

data:

message: >

🚨 *Smart Unified Alert*

🕒 {{ now().strftime('%H:%M:%S') }}

💨 Gas: {{ states('binary_sensor.gas_alarm') }}

🚶 Motion: {{ states('binary_sensor.living_room_motion') }}

🌡 Temperature: {{ states('sensor.living_room_temperature') }} °C

📍 [View on Map](https://maps.google.com/?q=13.9123,100.5211)