| ## Laboratory 9: GPS Integration (ESPHome To Home Assistant Map) | School of Applied Digital Technology |  |
| --- | --- | --- |
| Name: Name: | ID: ID: | Section: Section: |
| Date: | Due date: |  |

Objectives

- To integrate GPS (latitude and longitude) into smart sensors for location tracking.

- To display sensor positions on Home Assistant’s interactive map.

- To understand how location data can be used for monitoring and automation in IoT systems.

## Hardware and Software Requirements

| Component | Function |
| --- | --- |
| ESP32 DevKit | Main controller board |
| DHT11 Sensor | Measures temperature and humidity |
| MQ-2 Sensor | Detects gas concentration |
| LDR Sensor | Detects light intensity |
| PIR Sensor | Detects human motion |
| Fan (Relay-controlled) | Air circulation control |
| LED | Indicates brightness or darkness |
| Active Buzzer | Emits sound alarm on gas detection |
| Home Assistant + ESPHome | Visualization and automation control |

### Step 1 — Setup and Upload the ESPHome Code

- Use the following ESPHome YAML configuration (no modification needed):

Yaml:

sensor:

# 🌡️ DHT11

- platform: dht

pin: GPIO4

model: DHT11

temperature:

name: "Living Room Temperature"

id: temp_living

accuracy_decimals: 1

latitude: 19.9057

longitude: 99.8355

humidity:

name: "Living Room Humidity"

id: hum_living

accuracy_decimals: 1

latitude: 19.9057

longitude: 99.8355

update_interval: 10s

This configuration enables:

- Temperature and humidity monitoring via DHT11

- Gas detection via MQ-2 sensor

- Light intensity monitoring with LDR

- Motion detection using PIR

- Automatic fan operation when temperature exceeds 30°C

- LED control depending on light intensity

- Gas alarm triggering the buzzer above 0.40V

- Location tracking using predefined GPS coordinates

Step 2 — Adding GPS Coordinates for Each Sensor

- Each sensor and binary sensor includes latitude and longitude values.

Example:

Yaml:

temperature:

name: "Living Room Temperature"

id: temp_living

latitude: 19.9057

longitude: 99.8355

- Repeat this for all sensors (DHT11, MQ-2, LDR, PIR) to allow Home Assistant to plot them on a map.

Step 3 — Upload to ESP32

- Open ESPHome inside Home Assistant.

- Click INSTALL → WIRELESS (or via USB).

- Wait for build and upload to complete.

- When successful, ESPHome will show:

✅ Successfully compiled program.

✅ INFO Uploading /config/.esphome/build/project/firmware.bin

- The device should appear Online on the ESPHome dashboard.

### Step 4 — Add Map View in Home Assistant

- Add the following card in your Lovelace Dashboard (edit → Add card → YAML):

Yaml:

- type: map

title: "📍 Smart Room Sensor Locations"

default_zoom: 15

entities:

- entity: sensor.project_living_room_temperature

name: "Temperature"

- entity: sensor.project_mq_2_gas_level

name: "Gas Sensor"

- entity: sensor.project_ldr_light_level

name: "Light Sensor"

- entity: binary_sensor.project_living_room_motion

name: "Motion Sensor"

- entity: switch.project_living_fan

name: "Fan"

This displays pins on the Home Assistant map corresponding to each sensor’s coordinates.

Step 5 — Observation

- The map shows all devices’ GPS positions.

- Clicking a pin displays live sensor values.

- Automatic control works as follows:

  - 🌡️ Temperature > 30°C → Fan turns ON

  - 💨 Gas level > 0.40V → Buzzer turns ON

  - 💡 Light < 1.4V → LED turns ON

  - 🚶 PIR detects motion → “Motion Detected” status changes to ON

----- Have a good day -----